# Appendix C: Vocabulary

The following are terminology that is used in the discussion of the experiments

| Direct Current (DC) | A unidirectional flow of electrical current |
| --- | --- |
| Direct Current Voltage | A voltage that does not change polarity |
| DC Ammeter | A meter that measures direct current |
| DC Voltmeter | A meter that measures direct current voltage |
| Multimeter | A multi-functional meter that can measure current, voltage, resistance, Temperature, etc. |
| Photovoltaic (PV) | A method of generating electrical current from light |
| Photovoltaic Cell | A cell that generates an electrical current when illuminated by light– also called a solar cell |
| Photovoltaic Module | A group of photovoltaic cells connected together |
| Resistance | An electrical component that resists current flow turning electrical energy into heat. Used in circuits to control voltage and current. |
| Load | The device or devices connected to a voltage source |
| Short Circuit Current (ISC) | Current from a PV cell or module to a load of zero or extremely low resistance. |
| Open Circuit Voltage (VOC) | Voltage between the positive and negative terminals when there is no load connected to the device. (no current flowing) |
| IV Curve | A curve produced when plotting current (I) verses voltage (V) for a solar cell or solar cell module |
| Max power point | The voltage and current at which the maximum power is generated by a solar cell or module |
| Max power point voltage (Vmp) | The voltage at which the maximum power is generated by a solar cell or module |
| Max power point current (Imp) | The current at which the maximum power is generated by a solar cell or module |
| Global Irradiance (GHI) | Total solar radiation on a horizontal surface |
| Direct Normal Irradiance (DNI) | Total irradiance coming directly from the sun |
| Diffuse Horizontal Irradiance (DHI) | Irradiance from the dome of the sky, other than DNI that is incident on a horizontal surface |
| Zenith Angle | The angle between the sun and the normal to the horizontal surface |
| Angle of Incidence | The angle between the sun and the normal to a surface |